IMPORTANT: Read all instructions carefully before using this product.  Retain this owner’s manual for future reference. The specifications of this product may vary from this photo and, subject to change without notice.

2809.2-031920

3

1

PLEASE DO NOT RETURN THIS PRODUCT TO THE STORE.

STOP and contact customer service if you have any

questions regarding assembly or proper operation of the

Email us at:

Service@paradigmhw.com

Or call us at:

1-844-641-7920

Hours:

8:00 am to 5:00 pm (PST)

Monday thru Friday

TABLE OF CONTENTS

| SERVICE ------------------------------------------------------------------- | 2 |
| --- | --- |
| LABEL PLACEMENT --------------------------------------------------- | 3 |
| IMPORTANT SAFETY GUIDELINES ------------------------------- | 4 |
| ASSEMBLY PREPATION----------------------------------------------- | 5 |
| OVERVIEW DRAWING ------------------------------------------------ | 6 |
| PART LIST ----------------------------------------------------------------- | 7 |
| HARDWARE PACK ------------------------------------------------------ | 8 |
| PRODUCT DRAWING ------------------------------------------------ | 10 |
| STEP 1 ---------------------------------------------------------------------- | 11 |
| STEP 2 ---------------------------------------------------------------------- | 13 |
| STEP 3 ---------------------------------------------------------------------- | 15 |
| STEP 4 ---------------------------------------------------------------------- | 17 |
| STEP 5 ---------------------------------------------------------------------- | 19 |
| STEP 6 ---------------------------------------------------------------------- | 21 |
| WARRANTY --------------------------------------------------------------- | 23 |
| PARTS REQUEST FORM --------------------------------------------- | 24 |

1

SERVICE

IMPORTANT: FOR NORTH AMERICA ONLY

For damaged or defective product, questions, replacement parts or any other service support, please contact our customer service department by the below methods:

For The Best Service, please Email:

service@paradigmhw.com

Response Time: 1-2 Business Days

Emailing us with the information above will be the best method to receive a response during peak business hours

Website:

www.paradigmhw.com

Toll-Free:

1-844-641-7920 
 (8:00 AM - 5:00 PM Pacific Standard Time, Monday thru Friday)

Response time may vary via calling

Please have the following information ready when requesting for service:

- Your name

- Phone number

- Model number

- Serial number

- Part number

- Proof of Purchase

For damaged or defective product please contact our customer service before returning to the store.

Paradigm Health & Wellness, Inc.

1189 Jellick Ave.

City of Industry, CA 91748, USA

2

1

1

3

2

2

1

LABEL PLACEMENT

3

IMPORTANT SAFETY GUIDEILNES

WARNING: Before using this equipment you should consult with your physician to see if this workout equipment is appropriate for you.  Do not use this equipment without your physician’s approval if you have any of the following conditions or ailments:

  - Extreme obesity

  - Glaucoma, retinal detachment or conjunctivitis

  - Pregnancy

  - Spinal injury, Cerebral Sclerosis, or acutely swollen joints

  - Middle ear infection

  - High blood pressure, Hypertension, Recent stroke or Transient ischemic attack

  - Heart or circulatory disorders for which you are being treated

  - Hiatus hernia or Ventral hernia

  - Bone weaknesses including Osteoporosis, Unhealed fractures, Modularly pins, or Surgically implanted orthopedic supports

  - Use of anti-coagulants including Aspirin in high doses

The Maximum Static Weight Capacity for this product is 800lbs / 363kgs.

Read all the instructions carefully before assembling or operating this product. Retain this owner’s manual, do not remove any safety labels from the machine and keep the original purchase receipt for future reference.

This workout equipment is designed and built for optimum safety. However, certain precautions apply whenever you operate this exercise equipment.  Be sure to read the entire manual before assembling and operating this equipment.  Also, note the following safety instructions:

- Consult your physician or other health care professionals before using this equipment.

- Always wear proper exercise apparel when using this equipment. Use care when getting on or off the unit.

- If at any time you feel faint, light-headed, or dizziness while operating this equipment, stop exercising immediately and contact your physician. You should also stop exercising if you are experiencing pain or any kind of discomfort.

- Keep children and pets away from this equipment at all times.

- Only one person should use this equipment at a time.

- Make sure your equipment is correctly assembled before you use it. Be sure all the screws, nuts, and bolts are tightened prior to use. Check the screws, nuts, and bolts are tight on a weekly basis.

- Do not operate this or any exercise equipment if it is damaged.

- Wait 2 hours after eating before using this exercise equipment. If you get nauseous, stop exercising as soon as you feel queasy.

- This product should be placed on a flat surface when using. A mat or other covering material on the ground is recommended. For indoor use only, do not use outdoors or near water.

- Keep hands and feet away from any moving parts. Do not insert any objects into any openings on the equipment.

- Keep loose clothes, jewelry, limbs and long hair away from moving parts.

- Children under the age of 12 should not use this fitness equipment.

- Children from 12 to 18 should have adult supervision.

4

ASSEMBLY PREPARATION

Warning: It is highly recommended that you have assistance during the assembly of the strength equipment.

- Tools for assembly:

General tools you may need for the assembling this strength equipment:

    - Metric Allen Key Set

    - Metric Wrench Set and Adjustable Wrench

    - Flat Screwdrivers

    - Phillips Screwdrivers

    - Rubber Mallet

    - Silicone Spray Oil

- Insert bolts into the frame as illustrated in the drawing of each of the steps.

- Hand-tighten the bolts, nuts, and screw during assembly. Hand-tightening will allow for easily aligning the parts during assembly. Tighten all the hardware once the entire unit has been completely assembled.

- It is highly recommended that a professional installer assembles the strength equipment. But, with the proper assistance, the right tools, and strictly following the assembly steps, and given enough time; the assembly of the unit can be achieved without professional help.

- Thoroughly read each step before proceeding to assemble the parts of that step.

- To aid in the assembly of the equipment, the hardware pack (bolts, nuts, washer…etc.) have been presorted according to their corresponding steps. See the Hardware Pack page.

- When the equipment is fully assembled check all the functions for correct operation. Consult the manual if you experience any issues, or for further help please contact our service department. See page 2.

6

5

OVERVIEW DRAWING

6

6

| Part # | Description | Q'ty |
| --- | --- | --- |
| A | Upper Frame | 1 |
| B | Left J-Hook | 1 |
| C | Right J-Hook | 1 |
| D | Left Long Safety Catch | 1 |
| E | Right Long Safety Catch | 1 |
| F | Base Beam | 2 |
| G | Gusset Plate | 4 |
| H | Mounting Plate | 2 |
| I | Weight Plate Holder | 2 |
| J | Shaft | 2 |
| K | Landmine Attachment | 1 |
| L | U-Bracket | 1 |
| M | Rear Cross Bar | 1 |
| N | Upright Frame | 2 |
| 8 | Cable Pin | 2 |
| 9 | Elastic Cord | 2 |
| 10 | Long Rubber Pad | 2 |
| 11 | Short Rubber Pad | 2 |
| 12 | Knob | 1 |
| 13 | Bolt ￠12*M10*100 | 1 |
| 14 | Safety Pin ￠8*87 | 2 |
| 15 | Screw M6*10 | 12 |
| 32 | Hexogen Head Bolt M10*75 | 18 |
| 33 | Hexogen Head Bolt M10*20 | 2 |
| 34 | Spring Washer D15.4 *D10.2*T2.6 | 2 |
| 35 | Washer D30 *D10.5*T2.5 | 2 |
| 52 | Washer D24 *D13*T2.5 | 12 |
| 67 | Washer D20 *D10.5*T2.0 | 37 |
| 75 | Nylon Lock Nut M10 | 19 |
| 138 | Round End Cap | 8 |
| 294 | Spring Clip | 2 |
| 485 | Foot End Cap | 4 |

PART LIST

7

HARDWARE PACK

8

HARDWARE PACK

(294) Clip  2PCS

(12) Knob 1PC

(485) Foot End Cap 4 PCS

9

PRODUCT DRAWING

10

STEP 1

1A. Insert a Foot End Cap (485) into each side of the Base Frame (F).

2 - (485) Foot End Cap

1B. Attach the Rear Cross Bar (M) and two Mounting Plate (H) to the Base Frame (F) using:

4 - (32) Hexagon Head Bolt

10 - (67) Washer

4 - (75) Nylon Lock Nut

2 - (33) Hexagon Head Bolt

2 - (34) Spring Washer

Do not thoroughly tighten the hardware until instructed.

11

STEP 1

Wrench

2 PCS

(32) Hexagon Head Bolt 4PCS

(67) Washer

10PCS

(75) Nylon Lock Nut

4PCS

(33) Hexagon Head Bolt 2PCS

(34) Spring Washer

2PCS

(485) Foot End Cap

4PCS

12

STEP 2

2A. Attach the Upper Frame (A) onto both Upright Frames (N) by using:

4 - (32) Hexagon Head Bolt

8 - (67) Washer

4 - (75) Nylon Lock Nut

Note 1: The Upright Frames (N) have decals that say UP. The ends with the UP decal should be attached to the Upper Frame (A).

Note 2: There are two height installation options for the Upright Frames (N).

Do not thoroughly tighten the hardware until instructed.

13

STEP 2

Wrench

2 PCS

78 inch height option

81 inch height option

(32) Hexagon Head Bolt 4PCS

(67) Washer

8PCS

(75) Nylon Lock Nut

4PCS

14

STEP 3

3A. Attach four Gusset Plate (G) to the Base Beams (F) by using:

4 - (32) Hexagon Head Bolt

8 - (67) Washer

4 - (75) Nylon Lock Nut

3A. Attach two Upright Frames (N) to the four Gusset Plate (G) by using:

4 - (32) Hexagon Head Bolt

8 - (67) Washer

4 - (75) Nylon Lock Nut

Give the cage a shake so all the structure can settle and then thoroughly tighten ALL the hardware.

15

STEP 3

Wrench

2 PCS

(32) Hexagon Head Bolt 8PCS

(67) Washer

16PCS

(75) Nylon Lock Nut

8PCS

16

STEP 4

4A. Attach the Weight Plate Holder (I) onto the Upright Frame (N) by using:

2 - (32) Hexagon Head Bolt

2 - (67) Washer

2 - (75) Nylon Lock Nut

2 - (35) Washer

Give the cage a shake so all the structure can settle and then thoroughly tighten ALL the hardware.

17

STEP 4

Wrench

2 PCS

(35) Washer

2PCS

(32) Hexagon Head Bolt 2PCS

(67) Washer

2PCS

(75) Nylon Lock Nut

2PCS

18

STEP 5

5A. Assemble the Landmine Attachment (K) by using:

1 – (12) Knob

5B. Insert the Shaft (J) into the Gusset Plate (G) with the Landmine Attachment (K) and by using:

1 – (294) Clip

5C. Insert the second Shaft (J) into any the holes on the Base Beam (F) by using:

1 – (294) Clip

19

STEP 5

NOTE 3: The Shafts (J) can be used to hold squat resistance bands for adding additional resistance to a barbell squat.

*BANDS NOT INCLUDED

NOTE 4: The Landmine Attachment (K) should only be used in the 1 inch holes on the Gusset Plate (G).

(294) Clip

2PCS

(12) Knob

1PC

20

STEP 6

A. The two Right & Left Long Safety Catches (D) & (E) can be inserted into the holes along the Upright Frame (N). Always insert the Safety Pin (14) onto the Upright Frame (N) to lock them in place. Place them at the same level on the left and right side of the cage and at a comfortable height that will assist you during your workout. See Fig. A.

B. The two Right & Left J-Hooks (C) & (B) can be inserted into any of the holes along the Upright Frame (N). Place them at the same level on the left and right side of the cage and at a comfortable height that will assist you during your workout.

C. The Upright Frames (N) have “U” shaped markings above some of the holes to help mark which holes are at the same level across the frame of the cage.

D. Make sure the hooks of the Right & Left Long Safety Catches (D) & (E) and the Right & Left J-Hook (C) & (B) are wrapped around the Upright Frames (N) to lock them safety catches in place.

21

STEP 6

22

WARRANTY

MANUFACTURER’S LIMITED WARRANTY

Paradigm Health & Wellness warrants to the original purchaser that this product is free from defects in material and workmanship when used for the purpose intended, under the conditions that it has been installed and operated in accordance with Paradigm’s Owner’s Manual. Paradigm’s obligation under this warranty applies to the following:

COMPONENT           LENGTH OF WARRANTY (FOR HOME USE ONLY)

Structural Frame		   1 year (frame, welds)

Parts                      90 days (covers, internal mechanism, tension, wheels, knobs, hardware)

Wearable Parts             90 days (upholstery, foam, ball bearings, pulleys, belts, cables, Shocks)

Exclusions from Warranty Coverage:

Paradigm does not warrant against and is not responsible for, and no implied warranty shall be deemed to cover, any product failure, product malfunction, or damages attributable to:

1. Improper installation and/or failure to abide by Paradigm’s installation guidelines;

2. Use of this product beyond normal home use, or in an application for which it was not designed.

3. All exchanged parts and Products replaced under this limited warranty will become the property of Paradigm Health              and Wellness.

4. Damage caused by vandalism, accidents, inadequate maintenance, or by animals.

5. Any act of Nature (such as fire, flooding, snow, ice, hurricane, earthquake, lightning or other natural disaster), environmental condition (such as air pollution, mold, mildew, etc.), or staining from foreign substances (such as dirt, grease, oil, etc.).

6. Normal weathering due to exposure to sunlight, weather and atmosphere which can cause colored surfaces to, among other things, flake, rust, accumulate dirt or stains.

7. Improper operation, alteration, handling, storage, abuse or neglect of the product.

Paradigm, using its sole discretion, will either repair or replace free of charge any part(s) proven to be defective under normal home use.  Any repair or replacement shall provide no new warranty coverage, but shall retain only the remaining portion of the original product’s warranty. All exchanged parts and Products replaced under this limited warranty will become the property of Paradigm Health and Wellness. This warranty is offered only to the original purchaser and is not transferable.  Proof of original purchase is required.

Ordering Replacement Parts

Replacement parts can be ordered by emailing our customer service department:

Service@paradigmhw.com

Open Monday thru Friday 8:00 AM - 5:00 PM (PST).

When ordering replacement parts please have the following information ready:

1. Owner’s Manual

2. Model Number

3. Description of Parts

4. Part Number

5. Date of Purchase

23

PARTS REQUEST FORM

Paradigm Health & Wellness, Inc.

EMAIL THIS FORM WITH YOUR RECEIPT OF PURCHASE TO

Service@paradigmhw.com *

NAME:_____________________________________________________________________________________

ADDRESS:__________________________________________________________________________________

CITY:________________________ STATE:_____________ ZIP:_______________________________________

TELEPHONE:		(Day)______________________________________________________________________

(Night)_____________________________________________________________________

SERIAL#:___________________________________________________________________________________

MODEL#:___________________________________________________________________________________

PURCHASE DATE:___________________________________________________________________________

PLACE OF PURCHASE:_______________________________________________________________________

| PART # | DESCRIPTION | QTY |
| --- | --- | --- |

“YOUR ORDER WILL BE PROCESSED WITHIN 3 BUSINESS DAYS”

24

This form can also be faxed to #: 626-810-2166